FUSES

IGN 1 - Power for odd coils/injectors, 15A fuse

IGN 2 - Power for even coils/injectors, 15A fuse

IGN 3 - Switched ECM power, 15A fuse (PCM Blue Connector, Pin 19)

IGN 4 - Engine sensor power, 20A fuse (MAF, O2)

BAT 1 - Constant ECM power, 10A fuse (PCM Blue Connector, Pins 20 and 57)

RELAYS

Relay 1 -30Amp-Coil/Injector Power

Terminal 85   Hot in run and start (Coil Wire)

Terminal 86 	Ground

Terminal 30 	Direct to battery voltage

Terminal 87 	2 15A fuses (IGN 1-Bank 1 and IGN 2-Bank 2)

Relay 2 -40Amp-ECM/Sensor Power

Terminal 85 	Hot in run and start (Coil Wire)

Terminal 86 	Ground

Terminal 30 	Direct to battery voltage

Terminal 87   15A fuse (IGN 3-PCM Blue19) and 20A fuse (IGN 4-MAF/O2)

Relay 3 -30Amp-Fuel Pump Relay

Terminal 85 	PCM Red Connector Pin 9, Fuel Relay Control

Terminal 86 	Ground

Terminal 30 	Direct to battery voltage

Terminal 87 	20A fuse to fuel pump