- Disconnect battery ground cable, then drain cooling system. - Remove air cleaner assembly. - Remove accessory drive belts, then the fan and water pump pulley. - Disconnect all hoses from engine. - If equipped with automatic transmission, disconnect cooling lines from radiator. - Remove radiator and fan shroud. - Disconnect accelerator linkage and detent cable (automatic transmission models) from carburetor throttle lever. - If equipped with A/C, remove A/C compressor from mounting bracket and position aside. - Remove power steering pump, if equipped, and position aside. - Disconnect wiring harness from engine, then the fuel line from fuel pump. - Disconnect vacuum lines from intake manifold, then raise and support vehicle. - Drain crankcase, then disconnect exhaust pipes from exhaust manifolds. - On K and V models with automatic transmission, remove strut rods from motor mounts. - Remove flywheel or converter splash shield, then release A/T cooling lines along right pan rail. - Disconnect starter wiring and remove starter. - Disconnect both ends of the vacuum line to the transmission modulator. Set the line aside. - If equipped with automatic transmission, remove converter to flex-plate attaching bolts. - Support transmission with a suitable transmission jack, then remove bellhousing to engine attaching bolts. - Remove lower engine mount bracket to frame attaching bolts, then lower vehicle. - Remove hood. - Attach a chain to the lifting points (LF & RR) on the intake manifold. - Hook up the hoist (or chain fall in this case), take a strain and then slowly release the engine from its mounts. - Separate the engine/trans by drawing it slightly forward - towards the radiator support bracket. - When the locating pins on the bell housing disengage from the engine, lift it clear of the vehicle. - Reverse procedure to install.